Computer Network Mid-term Solution

  - b.

  - f.

  - b.

  - d.

  - b.

  - a.

  - d.

  - a.

2.1 a. transmission delay = L/R = (1,000 bytes × 8 bits/byte) / (1,000,000 bits/sec) = 0.008 seconds

b. I = La/R = (8,000 bits/packet × 30 packets/sec) / (1,000,000 bits/sec) = 0.24

queuing delay = IL/[R(1-I)] = (0.24 × 8,000 bits) / ( 1,000,000 bits/sec × (1-0.24)) = 0.0025 seconds

2.2 a. village’s capacity = (200 × 106 × 8 bits) / (12 × 60 × 60 sec) = 37037 bits / sec.

b. RTT = 24 hours. In the first round, the receiver cannot transmit the ACK. It can only transmit the ACK in the second round. So it takes the sender 24 hours to get the ACK.

c. Because three-way handshake is needed, the minimum time needed is 36 hours + T. Consider A is going to establish a TCP connection with B. At the end of first RTT, second rounds, A just receives its SYNACK, but it cannot transmit the third stage packet in the third round. At the beginning of the fourth round, A transmits the third stage packet. After T, that packet arrives at B and then the TCP connection is established.

d. Due to flow control, in order not overflow the C’s receive buffer, A cannot transfer 200 million bytes each time. According to TCP, C could tell A, by setting the 16-bit receive window field in the ACK, that there are at most 216-1 bytes free buffer available. So A will transfer no more than 216 – 1 bytes every time. Also note that A can upload 200 million bytes every 3 rounds. The utilization ratio of A’s capacity is (216 – 1)/(200,000,000 × 3) = 0.0109%.

e. In order to support fully utilization of A’s capacity, the receive buffer should be no less than 600 million bytes. So the A can upload 200million bytes every round.

2.3 a. application layer packet length: 64 + 8 = 72 bytes.

UDP segment header length: 8 bytes.

IP datagram header length: 20 bytes. (IPv4, no option field)

So, IP datagram length = 72 + 8 + 40 = 100 bytes.

b. packet rate = (16,000 bits/sec) / (64 × 8 bits/packet) = 31.25 packets/sec

packet rate = IP datagram rate

data rate = 31.25 datagrams/sec × (100 × 8) bits/datagram = 25000 bits/sec

2.4 a.

| step | N’ | D(B), p(B) | D(C), p(C) | D(D), p(D) | D(E), p(E) |
| --- | --- | --- | --- | --- | --- |
| 0 | A | 1, A | 3, A | 1, A | ∞ |
| 1 | AB |  | 3, A | 1, A | 2, B |
| 2 | ABD |  | 2, D |  | 2, B |
| 3 | ABDC |  |  |  | 2, B |
| 4 | ABDCE |  |  |  |  |

# A→B

# A→D

A→D→C

A→B→E

b.

| Destination | Next |
| --- | --- |
| B | B |
| C | D |
| D | D |
| E | B |