Peking University, School of Computer Science

Computer Networks

Spring 2022

Midterm Exam

Name:

Student ID:

Part 1 Multiple Choice

(5 points/question × 8 questions = 40 points)

For each question,

- There is ONLY ONE correct choice (1 point);

- Must show explanations or derivations that will lead to your choice (4 points).

Question 1.1

What is the function of flow control for a transport layer protocol? (1 pt)

- To prevent buffer overflow at the sender.

- To prevent buffer overflow at the receiver.

- To prevent buffer overflow at the next-hop receiver.

- To prevent buffer overflow at all intermediate routers along the end-to-end route from sender to receiver.

- To provide quality of service guarantees for a traffic flow through the network.

Why? (4 pts)

Answer:

Reason:

Question 1.2

Which of the following sources of delay in the Internet depends on traffic flows from other applications and hosts? (1 pt)

- Nodal processing delay.

- Packetization delay.

- Segmentation delay.

- Propagation delay.

- Transmission delay.

- Queuing delay.

Why? (4 pts)

Answer:

Reason:

Question 1.3

Which of the following is not true for cookies in HTTP? (1 pt)

- Cookies are supported by a header field in HTTP messages.

- The existence of cookies implies that HTTP is stateful.

- Cookies can be used to allow web applications to maintain state about clients.

- Cookies are stored on the client host.

Why? (4 pts)

Answer:

Reason:

Question 1.4

Consider an HTTP client retrieving multiple files from an HTTP server. Which scheme can retrieve all of the files in one round-trip time (RTT) plus the total transfer time of the files? (1 pt)

- Non-persistent connections.

- Persistent connections without pipelining.

- Persistent connections with pipelining.

- None of the above.

Why? (4 pts)

Answer:

Reason:

Question 1.5

What is the role of an authoritative name server in DNS? (1 pt)

- To provide the definitive IP address for any host name in the Internet.

- To provide the definitive IP address for any host name within its organizational domain.

- To provide authenticated IP addresses given a host name to any client host.

- To find a name server that can resolve any host name.

Why? (4 pts)

Answer:

Reason:

Question 1.6

Suppose an HTTP server is running on a host at port 80. Can we also run another application that uses UDP at port 80? (1 pt)

- Yes, because the port number is used by TCP and UDP to de-multiplex data, there is no conflict between the TCP and UDP applications using the same port number.

- Yes, as long as the UDP application also uses HTTP so de-multiplexing is not needed.

- No, since HTTP uses UDP, two applications running at port 80 are not allowed.

- No, because the port number is used to de-multiplex data, there will be conflict if two applications use the same port number.

Why? (4 pts)

Answer:

Reason:

Question 1.7

Why does TCP need a three-way handshake to establish a connection? (1 pt)

- The handshake establishes a client and server relationship.

- The handshake initializes TCP’s slow start mechanism used for congestion control.

- The handshake is needed for the two hosts to exchange their advertised window sizes needed to implement TCP’s flow control mechanism.

- The handshake provides a reliable transfer of the sequence numbers needed to implement TCP’s error control mechanism.

Why? (4 pts)

Answer:

Reason:

Question 1.8

Which of the following would be true if a virtual circuit network layer were used in the Internet? (1 pt)

- Routers in the backbone would have poor performance (compared to routers in the current Internet backbone) due to the amount of state information about virtual circuits that would need to be maintained.

- Routers in the backbone would have poor performance (compared to routers in the current Internet backbone) due to the time needed to process each datagram.

- Routers in the backbone could be simpler (compared to routers in the current Internet backbone) since setting up connections is simpler in a virtual circuit network.

- There would be no change since the Internet already uses a virtual circuit network layer.

Why? (4 pts)

Answer:

Reason:

Part 2 Question & Answer

Note:

Add any derivations, additional information or explanations in the answer to each question.

Question 2.1

(15 points) Consider the transmission delay and the queuing delay in a router buffer. Let I denote traffic density; that is

I=LaR

Suppose that the queuing delay takes the form

ILR(1−I)

for I<1.

Suppose packets are 1,000 bytes long and 30 packets arrive each second. The data rate of the link is 1,000,000 bits per second.

- What is the transmission delay for a packet? Give your result in seconds.

- What is the expected queuing delay for a packet? Give your result in seconds.

Question 2.2

(15 points) Consider the design of a new networking scheme called “BusNet,” where “bus” refers to an actual regional bus serving multiple villages in a rural area that does not have other mechanisms for data networking.

The bus stops at each village on its route and uses a short range, very high-data rate wireless connection to quickly transfer all IP datagrams for the local village to a local host of the village, and accepts all IP datagrams destined for all other locations. Note that the bus and the local host computer in each village only receive and send datagrams at a stop, and will not do any processing of the datagrams until after the current transfer completes.

An example route is shown below. The bus visits villages A through E in a circular route. It visits each village every 12 hours. The bus contains 200 million (200×106) bytes of storage for each village.

Assume that TCP’s timeout mechanism can tolerate a long waiting time so that TCP can work in this scenario.

- Without considering the requirement reliable data transfer protocols (e.g., TCP), what is the max data rate for a village to upload/transfer data to the bus in this system (a.k.a. the village’s capacity)? Give your answer in bits per second.

- What is the RTT between two local hosts of two villages in this system, for example as observed by TCP? Give your answer in seconds, minutes, or hours. (Discuss your reasoning with words and/or a diagram.)

- What is the absolute minimum time needed for TCP to establish a bidirectional connection, i.e., to complete a three-way handshake? Suppose the transit time from one village to the next one in the circle bus route is T.)

Question 2.3

(15 points) A VoIP system generates voice data at a rate of 16,000 bits per second. The system puts each 64-byte block of voice data into a separate packet for transmission. The application uses IP, UDP, and its own proprietary protocol which adds 8 bytes of data to each block of voice data.

- How long is each IP datagram sent for the application? Give your answer in bytes.

- What is the data rate for IP traffic carried by Link layer (or above Link layer)? Consider all headers and data for IP and above. Do not include Link layer headers in the calculated data rate.

Question 2.4

(15 points) Consider Dijkstra’s algorithm used with link-state routing for the graph shown below. The links are labeled with link costs.

- Find the shortest-path spanning tree rooted at node A (i.e., find the shortest paths from node A to all other nodes. Use the format XY...Z). (Show the tabular summary of Dijkstra’s algorithm.)

- Based on your results in part (a), specify the forwarding table at node A by completing the table below. Note that column “Next” is to specify the next node used by A to reach the specified destination.